Contents

Day 1: Basics of NMOS (drain current Ids vs drain to source voltage Vds)	2

Why do we need SPICE simulation?	2

SPICE simulation of std cell.	3

Study of Nmos transistor:	4

Surface inversion and VT	5

Impact of Vsb on VT	6

# Day 1: Basics of NMOS (drain current Ids vs drain to source voltage Vds)

## Why do we need SPICE simulation?

Study of P mos and-n mos transistor characteristic

- Voltage to Idsat graph.

- How leakage peaks during switching

## SPICE simulation of std cell.

- Why do we need it?

- All STA is built on SPICE simulation of fundamental devices in the std cell circuit.

  - Simulation helps to come up with the delay table calculation

  - The table contains Slew (rate of input change) vs output load

  - Depending upon different situation of the cell placement, the delays can be ‘looked-up’ on the table

  - If the table element doesn’t exists, then the value is extrapolated.

  - The source of these tables comes from spice simulation.

## Study of Nmos transistor:

- Nmos are isolated at the edge

- Bulk (psub) is tied to ground

- Gate is the most important control

- Applying voltage to Gate (Vgs)

- It builds a positive charge over the oxide cap plate.

- Which builds negative charge in the channel region

## Surface inversion and VT

- When the voltage applied is steadily increased, it leads to greater accumulation of negative dchange in the channel,

- In a P-substrate, there is a region created of negative change, and thus called inversion region.

- When inversion is at its highest,  it’s called threshold voltage.

- Once the majority positive charge is repelled from the channel at VT, further negative charges are absorbed from the N+ region

- This forms the continus channel

- Applying potential between Source and Substrate makes the diode more reverse biased

- This increases the channel width near source and substrate

## Impact of Vsb on VT

- In the presence of Vsb, additional voltage V1 is required to create the inversion region Vto+V1

- When Vds < Vgs -Vt device behave differently

- Voltage is not constant across the channel. There is a gradient

- From 0 to Vds